Task 01 Calculations
Group: B-1
Roll: SH-04, SH-22

Questions

- Using fTIM6_CLK=90MHz, determine PSC and ARR values that produce the following TIM6 overflow periods. For each target, show at least two valid (PSC, ARR) pairs and state which you would choose and why.

| Target Period | PSC | ARR | Actual Period |
| --- | --- | --- | --- |
| 1ms |  |  |  |
| 100µs |  |  |  |
| 500ms |  |  |  |
| 1s |  |  |  |

- For TIM3 generating 1kHz PWM on APB1 (fTIM3=90MHz): 
(a) Determine PSC and ARR.
(b) Compute the CCR1 value required for duty cycles of 25%, 50%, 75%, and 100%.
(c) Compute the CCR1 value for an arbitrary duty cycle D in terms of ARR.

- For TIM1 driving a WS2812B LED at fTIM1=180MHz, PSC = 0:
(a) Calculate ARR so that one-bit period equals 1250ns.
(b) Calculate CCR1 for a logic-1 bit (T1H ≈ 800ns).
(c) Calculate CCR1 for a logic-0 bit (T0H ≈ 400ns).
(d) Verify that your T1H and T0L values are within the WS2812B tolerance given in table.

| Symbol | Nominal | Tolerance | Meaning |
| --- | --- | --- | --- |
| T0H | 400ns | ±150ns | High time for a logic 0 bit |
| T0L | 850ns | ±150ns | Low time for a logic 0 bit |
| T1H | 800ns | ±150ns | High time for a logic 1 bit |
| T1L | 450ns | ±150ns | Low time for a logic 1 bit |
| RES | >50µs | --- | Reset latch: data line held LOW after last bit |

- The DWT cycle counter runs at fCPU=180MHz. Convert the following cycle counts to ns, µs, and ms: 3600, 90000, 9000000.

Solutions

- a) Given, 
fTIM6_CLK=90MHz
TPeriod=1ms

We know, 
TPeriod=ARR+1PSC+1fTIM6_CLK
Or, ARR+1PSC+1=TPeriod×fTIM6_CLK
Or, ARR+1PSC+1=0.001×90000000
Or, ARR+1PSC+1=90000

Set 1: ARR = 8999 and PSC = 9 so that ARR+1 times PSC+1 = 90000
Set 2: ARR = 44999 and PSC = 1, this pair also works

The second set is better because, it initially scales the frequency to 45MHz, which gives the tick-size almost 0.022µs so we can count lower periods later on.

b) Given, 
fTIM6_CLK=90MHz
TPeriod=100µs

We know, 
TPeriod=ARR+1PSC+1fTIM6_CLK
Or, ARR+1PSC+1=TPeriod×fTIM6_CLK
Or, ARR+1PSC+1=100×10-6×90000000
Or, ARR+1PSC+1=9000

Set 1: ARR = 4499 and PSC = 1
Set 2: ARR = 999 and PSC = 8

The first pair is suitable as it gives tick at almost 0.022µs time, so the chunk is short and accurate for flexible counting.

c) Given, 
fTIM6_CLK=90MHz
TPeriod=500ms

We know, 
TPeriod=ARR+1PSC+1fTIM6_CLK
Or, ARR+1PSC+1=TPeriod×fTIM6_CLK
Or, ARR+1PSC+1=0.5×90000000
Or, ARR+1PSC+1=45×106

Set 1: ARR = 59999, PSC = 749
Set 2: ARR = 65502, PSC = 687

In this case also, Set 1 is better for fine-tuning later on and gives the exact timing. But in Set 2, it has better resolution (8.33µs vs 7.64µs) but doesn’t hit the target time (500.4ms almost). So, both have tradeoffs but I would choose Set 1 for this case as there are very little benefits from Set 2.

d) Given, 
fTIM6_CLK=90MHz
TPeriod=1s

We know, 
TPeriod=ARR+1PSC+1fTIM6_CLK
Or, ARR+1PSC+1=TPeriod×fTIM6_CLK
Or, ARR+1PSC+1=1×90000000
Or, ARR+1PSC+1=90×106

Set 1: ARR = 65529 and PSC = 1373
Set 2: ARR = 59999 and PSC = 1499

The second pair here is better because of the precision of the Target Period. First pair gives the time slightly more, but their resolution difference is about 1.4µs, so it is better to choose Set 1 for precise operation.

Finally, all pairs used here create the same target period mentioned in the table:

| Target Period | PSC | ARR | Actual Period |
| --- | --- | --- | --- |
| 1ms | 1 | 44999 | 1ms |
| 100µs | 1 | 4499 | 100µs |
| 500ms | 749 | 59999 | 500ms |
| 1s | 1499 | 59999 | 1s |

- a) Given, 
fTIM3=90MHz
fPWM=1kHz

We know, 
fPWM=fTIM3ARR+1PSC+1
Or, ARR+1PSC+1=fTIM3fPWM
Or, ARR+1PSC+1=900000001000
Or, ARR+1PSC+1=90000

For this PWM output, ARR = 44999 and PSC = 1 is a good choice

b) From (a), ARR = 44999

∴For Duty Cycle=25%, CCR1= D100×ARR+1=11250
For Duty Cycle=50%, CCR1= D100×ARR+1=22500
For Duty Cycle=75%, CCR1= D100×ARR+1=33750
For Duty Cycle=100%, CCR1= D100×ARR+1=45000


c) For any arbitrary CCR1 value,

CCR1=D100×ARR+1, where D=Duty Cycle in Percentage

- a) Given, 
fTIM1_CLK=180MHz
∴TTIM1_CLK=1180000000≈5.56ns

Now,
Number of Ticks ≈ 12505.56≈225 and ARR ≈ 225 - 1 ≈ 224 (Ans.)


b) Given,
TT1H≈800ns
TPeriod=1250ns
From (a), ARR ≈ 224

∴CCR1=TT1HTPeriod×ARR+1≈8001250×225≈144 (Ans.)


c) Given,
TT0H≈400ns
TPeriod=1250ns
From (a), ARR ≈ 224

∴CCR1=TT0HTPeriod×ARR+1≈4001250×225≈72 (Ans.)


d) From (a), TTIM1_CLK≈5.56ns

For CCR1≈144,
T1H≈(144×5.56)ns≈800.64ns≈800ns

Again,
For CCR1≈72,
T0H≈(72×5.56)ns≈400.32ns≈400ns

This verifies that the CCR1 values lie between the threshold of T0H and T1H i.e. 250-550ns and
650-950ns. For the LOW values,

T1L≈1250-800ns≈450ns[Between the 300-600ns threshold]
T0L≈1250-400ns≈850ns[Between the 600-900ns threshold]

Finally, all values cover the tolerance threshold mentioned in the table.

- Given,

fCPU=180MHz

- For Cycle=3600,

tns=Cycle×1000fCPU(MHz)=3600×1000180=20000ns

tμs=CyclefCPU(MHz)=3600180=20μs

tms=CyclefCPU(MHz)×1000=3600180×1000=0.02ms (Ans.)

- For Cycle=90000,

tns=Cycle×1000fCPU(MHz)=90000×1000180=500000ns

tμs=CyclefCPU(MHz)=90000180=500μs

tms=CyclefCPU(MHz)×1000=90000180×1000=0.5ms (Ans.)

- For Cycle=9000000,

tns=Cycle×1000fCPU(MHz)=9000000×1000180=50000000ns

tμs=CyclefCPU(MHz)=9000000180=50000μs

tms=CyclefCPU(MHz)×1000=9000000180×1000=500ms (Ans.)

========== 0 ==========