UNIVERSITY OF DHAKA

Department of Computer Science & Engineering

LAB REPORT

| Semester | 2nd Year, 2nd Semester 2026 |
| --- | --- |
| Course Code and Title | CSE 2206 — Microcontroller & Embedded System |
| Assignment | 02 |
| Task | 7 |
| Topic | Real-World Applications & SDG Reflection |
| Group | B1 |
| Report Prepared By | Syed Muhtasim Alam Apon(SH-04) |
| Date Submitted | May 11, 2026 |

# PART A — Timer-Based Applications in Modern Embedded Systems

Timers are the fundamental peripheral of every modern embedded system. The three safety-critical, commercially deployed systems examined below represent absolute timer functions

## A.1 — Industrial Motor Drive (Variable Frequency Drive)

## Timer Function Used:

Industrial Variable Frequency Drives (VFDs) control three-phase induction motors in factories, water treatment plants, and textile mills across Bangladesh. The STM32 timer generates a sinusoidal PWM waveform to synthesise variable-frequency AC output from a DC bus. A second timer channel performs frequency measurement on the motor's tachometer pulse train to implement closed-loop speed control. PWM generation , delay , accuracy measurement - functions mainly used .

Timing Accuracy Requirement:

The PWM dead-time insertion between high-side and low-side gate pulses must be controlled under a protocol to within ±100 ns. Without this, shoot-through (transition condition interval conduction of both switches) destroys the IGBT power stage instantly.

Consequence of 10% Timing Error:

A 10% stretch of the 50 µs PWM period shifts the synthesised AC frequency from 50 Hz to 45 Hz. The motor runs 10% slower — acceptable in some pumps, catastrophic in a CNC spindle or textile loom where product tolerances measurements are specified. More critically, a 10% error in dead-time (e.g. 1.1 µs instead of 1.0 µs) causes glitches and notches that induce motor winding insulation breakdown over weeks of operation.

## A.2 — Dialysis Machine (Haemodialysis Pump Controller)

## Timer Function Used:

A haemodialysis machine circulates  blood at a prescribed rate (typically 200–400 mL/min) through an osmosis ultrafilter. The blood pump is driven by a DC motor whose speed is set by a PWM duty cycle generated by a hardware timer. A separate timer period controls the ultrafiltration rate — the volume of fluid removed per hour. A watchdog timer independently takes reading from the firmware: if the pump ISR fails to kick it within 2 seconds, the pump halts and an alarm fires to protect the patient

Timing Accuracy Requirement:

Flow-rate  must be calculated accurately within ±5% per IEC 60601-2-16. To achieve this, the timer period controlling motor speed must be stable to better than 1% across the 0–40°C clinical operating range — requiring a crystal oscillator source(a HSE oscillator for external source).

Consequence of 10% Timing Error:

A 10% fast pump over-delivers blood volume, causing haemoconcentration and potentially fatal clot formation in the circuit. A 10% slow pump under-clears uraemic toxins, leaving the patient symptomatic and at risk of hyperkalaemia-induced cardiac arrest. Regulatory bodies (US FDA, EU MDR) classify this as a Class III life-threatening defect.

## A.3 — Metro Rail Traction Control System (DHAKA MRT Line-6)

Timer Function Used:

The Dhaka MRT Line-6 uses embedded controllers that drive the three-phase motors of each rail car. The traction MCU generates PWM at a switching frequency of 2.5 kHz to control torque and speed. A timer input-capture channel reads pulse-per-revolution signals from the axle encoder to determine speed, reporting a closed-loop torque controller. A precise timer interrupt sequence executes the regenerative braking ramp  within a ±5 ms window to prevent contact wire overvoltage.

Timing Accuracy Requirement:

The SVPWM carrier period must be stable to ±2 µs. The axle encoder pulse measurement must address speed to ±0.1 km/h for the Automatic Train Protection (ATP) system to enforce speed limits reliably.

Consequence of 10% timing error:

A 10% error in the PWM period shifts the inverter output frequency, causing the traction motor to produce incorrect torque — the train accelerates or decelerates at the wrong rate. A 10% error in the regenerative braking timer overvoltages the line, tripping the substation and stranding all trains in the affected section.

| System | Timer Function | Accuracy Required | 10% Error Consequence |
| --- | --- | --- | --- |
| Industrial VFD | PWM + dead-time control | ±100 ns dead-time | IGBT shoot-through; motor failure |
| Dialysis Pump | PWM speed + watchdog | <1% flow rate | Over/under-dialysis; patient fatality risk |
| MRT Traction | SVPWM + encoder capture + brake ISR | ±2 µs PWM; ±0.1 km/h speed | ATP violation; emergency stop; substation trip |

PART B — WS2812B and Smart Lighting in Smart Cities

Bangladesh stands at an inflection point, continuously facing the challenge of keeping energy affordable, keeping cities lively, and its commitments under the United Nations Sustainable Development Goals.

## B.1 — PWM Duty-Cycle Control and Energy Efficiency: SDG 7 & SDG 11

| SDG 7 | Affordable and Clean Energy — Ensure access to affordable, reliable, sustainable and modern energy for all. |
| --- | --- |

| SDG 11 | Sustainable Cities and Communities — Make cities and human settlements inclusive, safe, resilient and sustainable. |
| --- | --- |

The WS2812B LED, driven by an STM32 PWM timer, replaces this with adaptive intensity control via 8-bit duty-cycle programming (0–255 per R/G/B channel).The energy saving mechanism is direct: power usage scales linearly with duty cycle. At 50% duty cycle, a WS2812B string draws half the current of full brightness — and noteworthy, it also emits half the heat. For Bangladesh's SDG 11 target of sustainable, resilient cities, this reduction in thermal load across thousands of street fixtures meaningfully lowers the urban heat island effect, reducing the city's nocturnal temperature by an estimated 0.3–0.8°C in dense deployments — a measurable contribution to climate change.

The energy efficiency opportunity lies in adaptive dimming. A WS2812B-based fixture driven by an STM32 node can implement a lighting profile:

- Full brightness (duty ≈ 200–255) from 18:00–22:00 during peak pedestrian hours

- Reduced brightness (duty ≈ 80–120) from 22:00–05:00 during low-traffic hours

- Minimum safe illumination (duty ≈ 30–50) from 02:00–04:00 when footfall drops near zero

- PIR sensor trigger: burst to full brightness within 200 ms, decay over 90 seconds

A simple three-phase dimming schedule reduces energy consumption by 45–55% compared to a fixed-brightness installation.  At Bangladesh's industrial electricity tariff of approximately Tk 9/kWh, this represents annual savings of Tk 4.3 million (approximately USD 39,000) for a single urban district — directly advancing SDG 7's affordability mandate. PWM output duty cycle embodies the self-sustaining energy use. A 10,000 installment in Dhaka saves an estimated 480 MWh/year compared to fixed sodium lamps, reducing CO2 emissions by approximately 330 tonnes annually.

The SDG 11 dimension is equally compelling. Consistent, well-organised street illumination improves pedestrian safety and reduces nighttime crime — both measured outcomes in sustainable city metrics. Colour temperature can be adjusted .  Solar energy integration is architecturally compatible. The stored energy powers the WS2812B fixtures at night, making the entire lighting node energy-autonomous from the grid.

## B.2 — STM32  CAN Bus Adaptive Street Lighting Network Architecture

A network of STM32F446-based nodes interconnected over a CAN bus provides a robust, electrically-noise-immune backbone for citywide adaptive lighting. CAN bus (ISO 11898) operates at up to 1 Mbit/s on a differential twisted-pair, is inherently multi-master, and provides hardware CRC error detection — properties that make it the industry standard for automotive and industrial control networks operating in electrically noisy environments. A street-lighting pole presents exactly this environment: heavy inductive loads nearby, motor-driven traction systems on adjacent roads, and long cable runs susceptible to common-mode noise.

The proposed architecture is as follows:

| Layer | Component | Role |
| --- | --- | --- |
| Sensing | Ambient Light Sensor (BH1750 or TSL2591) + PIR Motion Sensor | Measure lux levels and pedestrian presence; feed data to STM32 via I2C |
| Processing | STM32F446 Node MCU | Run dimming algorithm; drive WS2812B chain via TIM1 PWM + DMA; transmit CAN frames |
| Network | CAN Bus (ISO 11898), up to 500 m per segment | Carry node status, sensor readings, dimming commands, and OTA firmware deltas |
| Gateway | Raspberry Pi / industrial gateway with CAN-USB | Aggregate data; execute city-level scheduling policy; report to cloud dashboard |
| Cloud | MQTT broker + dashboard | Municipality operator UI for zone control, fault alerting, and energy analytics |

Each pole-mounted STM32F446 node monitors BH1750 via I2C and a PIR motion detector . The node adjusts its WS2812B chain duty cycle locally using TIM1 PWM + DMA. Every 10 seconds it broadcasts a CAN frame containing: lux reading, PIR state, current duty cycle, battery voltage, and node health.  Nodes without a gateway heartbeat for 60 seconds fall back to a stored default schedule, ensuring continuous illumination during gateway maintenance.

## B.3 — Firmware Design Choices for a 10-Year Continuous Deployment

A deployment expected to run continuously for ten years without human intervention demands firmware architecture designed from the ground up around reliability, power efficiency, and self-healing.

### Interrupt-Driven vs. DMA

- DMA for WS2812B: TIM1 output-compare register is fed by DMA2 — the CPU is completely uninvolved during LED data transfer, eliminating jitter and enabling simultaneous sleep.

- Interrupt-driven sensors: BH1750 and PIR use I2C interrupt + EXTI — no polling, minimal CPU wake time.

- DMA decouples the CPU from the bit-stream, allowing the core to sleep in low-power mode (SLEEP or STOP) while the DMA controller autonomously transfers the 24-bit colour data for each LED in the chain.

- Interrupt-driven I2C for the sensor reads is acceptable — sensor polling happens only once per second, and the CPU can service the interrupt within the 400 kHz I2C clock period budget easily.

### Low-Power Sleep Modes

- In  SLEEP mode (peripherals active, CPU halted) with 16 MHz clock, current falls to roughly 8 mA. In STOP mode (all clocks gated, SRAM retained) current drops to approximately 100–300 µA. The correct approach for this deployment is: run the sensor .

- STOP mode: After each 10-second CAN broadcast cycle the MCU enters STOP mode (~150 µA). RTC alarm and PIR EXTI are the only wake sources. Average current <0.3 mA over the full cycle.

### Watchdog Timers

- The Independent Watchdog (IWDG), clocked from the 32 kHz LSI oscillator, must be kicked every 2 seconds by the main task loop. If any firmware fault — stack overflow, infinite loop, peripheral lockup —can be prevented.

- IWDG watchdog : Independent of all system clocks. Any firmware hang resets the node within 2 seconds, restoring illumination automatically.

- Backup SRAM reboot counter: If the node reboots >5 times per hour, it locks to 50% white (safe fallback) and sends a CAN alarm — preventing a firmware bug from leaving a street dark.

| Firmware Choice | Selected Approach | Why It Matters for 10-Year Life |
| --- | --- | --- |
| WS2812B Driving | DMA (TIM1 CH1 via DMA2 Stream 6) | Zero CPU involvement; no jitter risk; CPU sleeps during transfer |
| Sensor Polling | Interrupt-driven I2C + RTC wake | Sensor read in <5 ms; CPU in STOP 99.5% of cycle |
| Network Keepalive | CAN FD at 500 kbit/s, 10 s broadcast | Hardware CRC catches bit errors; no polling of gateway required |
| Primary Watchdog | IWDG, 2 s timeout, LSI clock | Independent of main clock; resets on any firmware hang |
| Secondary Watchdog | WWDG, 50 ms window, PCLK1 | Catches CAN driver lockup specifically |
| Boot Fault Detection | Backup SRAM reboot counter | Limits damage from firmware reboot loops; fallback illumination ensured |

# PART C — Design Challenge: Solar-Powered IoT Greenhouse Node

The architecture presented below assigns each functional task to a dedicated STM32F446 timer peripheral, justifies every hardware abstraction decision, and describes a lightweight time-sliced loop that coordinates all tasks without preemption.

## C.1 — Timer Peripheral selection for each Scenario

| Subsystem | Peripheral | Mode | Reason |
| --- | --- | --- | --- |
| RGB LED (WS2812B) | TIM1 CH1(APB2, 180 MHz) | PWM + DMA2Stream 6 | DMA drives bit-stream autonomously; CPU sleeps during transfer. LED colour encodes: Green=normal, Yellow=warning (high temp/gas), Red=fault (pump/sensor fail). |
| Servo-controlled ventilation(window + door open/closefor airflow management) | TIM3 CH1(APB1, 90 MHz) | PWM Output50 Hz, 1–2 ms | RC servo protocol (20 ms period, 1–2 ms pulse) has no sub-µs timing requirement. HAL_TIM_PWM_ConfigChannel() is portable and reduces development time. Window/door position maps linearly to CCR value; airflow sensor feedback adjusts target angle. |
| BME280 sensor(Temperature, Humidity,Pressure response timing) | TIM2 CH2(32-bit, APB1) | Input Capture | BME280 I2C transaction and conversion-ready timing must be measured precisely to compute sensor response latency. TIM2's 32-bit range covers the full 113 ms BME280 measurement cycle without overflow. Input-capture ISR timestamps start and end of conversion; CPU computes response time from delta. |
| UART telemetry(sensor data every 30 s) | TIM6 Basic Timer+ USART2 DMA | Update IRQ@ 30 s period | TIM6 has no output pins — ideal as a pure IRQ source. 30 s auto-reload wakes MCU from STOP, triggers HAL_UART_Transmit_DMA (non-blocking). MCU re-enters STOP on DMA TC callback. |
| Little Power Consumption | IWDG(LSI clock, ~32 kHz),Low Power Mode | 4 s timeout | Independent of all system clocks; kicks every 2 s from main loop. Unrescued hang resets node within 4 s.Low Power Mode ensures power consumption , watchdog timers resets mcu if it goes to any faulty state. Using of DMA ensures very low latency and time consumption. |

## C.2 — HAL vs. Bare-Metal Justification

The decision between STM32 HAL (Hardware Abstraction Layer) and direct register manipulation (bare-metal) must be made per-subsystem based on three factors: timing criticality, code portability requirement, and maintenance cost.

### WS2812B Driver — Bare-Metal

In bare-metal, the DMA transfer-complete ISR directly writes the next frame's data pointer and length to the DMA stream control register in less than 10 clock cycles. This determinism is impossible to guarantee through HAL's callback chain. Additionally, the HAL's timer DMA implementation for TIM1's burst mode is under observations and has known errata interactions on the F446. Bare-metal register access, guided by the reference manual, eliminates all ambiguity.

### Servo PWM Driver — HAL

RC servo control at 50 Hz with millisecond-range pulse widths has no timing criticality beyond ±10 µs — easily met by HAL. Using HAL_TIM_PWM_ConfigChannel() and HAL_TIM_PWM_Start() reduces development time from hours to minutes and makes the pulse-width calculation (duty_cycle = (pulse_µs / 20000) × ARR) readable and portable. If the project is later ported to an STM32G4 or STM32H7, the HAL call sequence is identical. This is exactly the use case HAL was designed for.

### BME280 Sensor — Bare-Metal

BME280 sensor reading is timing-critical at the bit-decode stage. HAL_TIM_IC_SensorCapture() fires through the HAL interrupt framework with 3–5 µs of callback latency jitter on a busy system. risking misclassification. Direct register access in the TIM2 CC2 ISR — reading CCR2 immediately on entry — keeps latency below 500 ns and decodes bits correctly every time.

### UART Telemetry — HAL + DMA

UART frame transmission is not time-critical at the microsecond scale; a 30-second period has a scheduling jitter budget of tens of milliseconds. HAL_UART_Transmit_DMA() handles the UART DMA setup, handles the half-transfer and full-transfer interrupts, and calls a completion callback — all without blocking the CPU. This is the canonical use of HAL and saves significant development effort with zero performance penalty. The completion callback simply sets a flag that the super-loop checks to confirm transmission before re-entering STOP mode.

## C.3 — Multi-Task Coordination Without an RTOS

The architecture uses a cooperative time-divided super-loop driven by the SysTick 1 ms interrupt. Each task registers a period and a function pointer in a static task table. The SysTick ISR increments a millisecond counter and sets a per-task 'ready' flag when the elapsed time exceeds the task's period. The main loop polls the flag table and dispatches tasks in priority order. Because all tasks are cooperative and non-preemptive, there are no race conditions, no mutexes required, and no stack sizing challenges — the entire architecture fits in a few hundred bytes of RAM.

The task table for this greenhouse node is:

| Task | Period | Priority | Blocking Constraint |
| --- | --- | --- | --- |
| WS2812B LED Update | 100 ms | 1 (Highest) | Non-blocking: DMA-driven; ISR sets frame-done flag; task only primes next frame buffer |
| DHT22 Sensor Read | 2 s | 2 | Non-blocking: Input-capture ISR fills bit buffer; task decodes completed buffer |
| Servo Position Update | 20 ms | 3 | Non-blocking: HAL_TIM_PWM writes CCR only; returns in <5 cycles |
| UART Telemetry TX | 30 s | 4 | Non-blocking: HAL DMA TX; completion callback sets tx_done flag; MCU returns to STOP |
| IWDG Kick | 1 s | 5 (Background) | Non-blocking: single register write; must never be missed |

Power management:

at the bottom of the super-loop, if no task is ready and no DMA transfer is active, the MCU enters STOP mode (<150 µA). A DMA transfer in progress triggers SLEEP mode only (clocks active, CPU halted). Wake sources: RTC alarm (30 s for UART telemetry), TIM6 update IRQ, EXTI from PIR or water-level threshold, and IWDG. The combination of DMA-driven peripherals, interrupt-based sensors, and aggressive STOP-mode usage eliminates all software busy-wait delays, keeping average node current below 0.5 mA — essential for solar-battery-powered operation with no grid backup.

This architecture delivers deterministic, auditable, maintainable firmware without the overhead or failure modes of a preemptive kernel — the correct engineering choice for a decade-long field deployment in rural Bangladesh.